TAPECHECK is a program designed to quickly and inexpensively test the

data reliability of a magnetic tape.  It does this by writing blocks

of data on the entire tape, rewinding the tape, and reading the entire

tape back in.  Data blocks which do not match the original data pattern

or block length are announced along with the approximate position on the tape.

THIS PROGRAM OVERWRITES ALL THE DATA ON THE TAPE.  Please do not attempt

to check a tape that has valuable information on it.

The check performed by this program is not as good as is available from

any hardware tape evaluator.  The errors that this program reports might

conceivably be caused by the tape drive or some other factor.  However,

this program should run on virtually any VMS configuration with a mag tape.

It has been tested on various combinations of (750/780),(TU77/TU78),(2.5/3.0).

The author would be pleased to hear of any bugs found or improvements made.

RESTRICTIONS:

If you have a tape drive that does not run at 800, 1600 or 6250 BPI,

the program will complain and quit.  If the density you run at does not

match the density the system thinks you are at, the program may run but

will quote lengths incorrectly.

The program does terminal QIOs (sorry) and thus will only run from a terminal

or a command procedure as long as logical name TT points at a terminal.

What this means is you cannot easily run it in batch.

TO USE THE PROGRAM:

1) Mount the tape /FOREIGN at desired density* with logical name SCRATCH.

*	If the tape has been previously used at

another density, the tape drive may set

the density for you.  If this is a problem,

INIT the tape first at the required density.

2) Run the program.

Sample Output:

CHECKING TAPE AT 1600 BPI

THE TAPE IS ABOUT 2332 FEET LONG

A TAPE ERROR OCCURRED AT ABOUT 2301 FEET

A TAPE ERROR OCCURRED AT ABOUT 2302 FEET

2 TAPE ERRORS OCCURRED IN 2332 FEET

3) Dismount the tape.

There is reasonable documentation within the source file for finding out

how it works if anyone is interested.  This is a later release of the

same program submitted to an earlier DECUS tape, I think Fall 82.

The tape check can be made more stringent by changing the program to

inhibit retries on writes. (See source).  Anybody with a hardware tape

evaluator might run some known bad tapes past this program and let

me know the results.

This program writes on about 95% of the tape surface, the rest goes to

interrecord gaps.  Obviously the gaps are not checked for data reliability.

It is difficult to do much better than that on a standard tape drive.

OCTOBER 14, 1982

JIM LELLMAN

G. D. SEARLE & CO.

RESEARCH COMPUTING SERVICES

4901 SEARLE PARKWAY

SKOKIE IL, 60077