May 19, 1983

The patches and programs in  this  submission  are  for

running  the KMC-11 debugging tools designed for the PDP-11.

By modifying compatibility mode programs to include resident

common  program  sections  and installing global sections in

the VAX/VMS system it is possible for these programs to  map

to  the  I/O  pages (UNIBUS address space).  The two Digital

Equipment Corporation proprietary programs KMCLDR and  KMCDA

are patched to include these sections and address them as if

they are the I/O pages.

BUSCOM          ______

BUSCOM is used to create or delete two permanent  group

global sections in memory.  The two sections UBUS0 and UBUS1

are located at the UNIBUS zero and UNIBUS one address  space

on  an  11/780.   The  program  should be run before the KMC

loader or debugger is run.  All members in  the  same  group

can  then  run  the  loader  or debugger.  The sections that

BUSCOM creates will stay resident until the system is booted

or  until  the  sections  are  deleted.   Two privileges are

required to run this program, map to specific physical pages

(PFNMAP)  and  create  permanent  global  sections (PRMGBL).

These will be enabled by the program for the duration of its

execution.

To verify that the global sections have  been  created,

issue  the  command "MCR INSTALL /GLOBAL".  The two sections

will appear as "UBUS0_001" and "UBUS1_001" under the listing

of  "Group Global Sections" with the group number from which

the program BUSCOM was run.  Change mode to kernal  (CMKRNL)

privilege is required for this command.

BUSCOM asks a single  question,  whether  you  wish  to

create,  delete  or  exit.   Either  case  single  character

answers will suffice and no acknowledgement is made  if  the

operation  is  successful.  If an error occurs then a status

message will be printed.

UBUS0 and UBUS1          _____ ___ _____

The  files  UBUS0.*  and  UBUS1.*  are  used  when  the

compatibility mode loader and debugger are built so that the

task builder (TKB) can use the symbol tables for  reference.

Both  the  tasks  (UBUS0.TSK  and  UBUS1.TSK) and the symbol

tables (UBUS0.STB and UBUS1.STB)  must  be  present  in  the

directory when building the loader and debugger.  The symbol

tables and  tasks  are  necessary  because  the  loader  and

debugger  are built so that they reference a resident common

data area.

If it becomes necessary  to  rebuild  either  of  these

tasks   command  files  UBUS0BLD.COM  and  UBUS1BLD.COM  are

provided.

Page 2

PROPRIETARY SOURCE          ___________ ______

The loader and debugger  are  named  KMCLDR  and  KMCDA

respectively.   The  two programs are proprietary to DEC and

therefore they could not be  included  in  this  submission.

The  patches  that  are  described  here were applied to the

following distribution from DEC, an RK05 pack dated  5/30/78

with the label:

AN-C914A-SC

KMC-11 TOOLS V1 RK05

The loader has the  ".ident"  of  /X01/  and  the  last

change  is  from "J OLAPURATH 8/25/76".  The debugger has an

".ident"   of   /V0001A/   and    last    modification    of

23-SEPTEMBER-77  by DAVID J.  MITTON.  I believe the current

product number is QJ633 "KMC11 TOOLS".

Also included in this distribution  was  a  macro  file

(KMCMAC.MAC)  which  defines  all the KMC-11 mnemonics and a

file which is used to set up break points in the KMC-11 code

(BRKPNT.MAC).

APPLYING PATCHES          ________ _______

The two source language patch files are KMCLDR.SLP  and

KMCDA.SLP.   They  should  be  invoked  as DCL command files

("@KMCLDR.SLP" and "@KMCDA.SLP").  They  include  checksums,

and expect the PDP-11 versions of the loader and debugger to

be in  the  current  directory.   A  differences  comparison

listing  was generated and these are in the files KMCLDR.DIF

and KMCDA.DIF.

Two files KMCLDRBLD.COM and KMCDABLD.COM  are  provided

for assembling and linking the loader and debugger.

RUNNING KMCLDR AND KMCDA          _______ ______ ___ _____

The only additional input to the programs beyond asking

for  the  CSR  of the KMC is that both will ask for a UNIBUS

number.  This should be answered with either a "0" or a "1".

The  CSR  question  should be answered as normally with a 16

bit value.

EXAMPLE PROGRAM          _______ _______

The files DRECHO.* contain an  example  KMC-11  program

that will take input from a DR-11K and echo it back out.  It

will continue until an interrupt and then it will perform an

internal loop.  It must be stopped manually.

The program can be loaded using KMCLDR, and then  after

invoking KMCDA, a "G"o command will start it.

Page 3

ASSEMBLING AND LINKING          __________ ___ _______

To assemble and link KMC-11 source files a command file

is  provided  with  the  name  KMCBLD.COM.   It  expects the

KMCMAC.MAC (supplied with the debugging tools) macros to  be

in the current directory.

CLN TECO MACRO          ___ ____ _____

The CLN.TEC file is used to clean up  assembly  listing

files.  The programs should be assembled with the KMCMAC.MAC

file provided and with a ".LIST MEB" directive.   The  clean

macro will remove all the expansions and leave the addresses

and machine code.  This is extremely helpful for  debugging.

To  invoke  the  TECO macro you must already be TECO'ing the

listing file and invoke the command "EICLN$$".

COMMENTS AND SUGGESTIONS          ________ ___ ___________

I welcome any comments or suggestions:

Thomas J. Linscomb

Computation Center

The University of Texas at Austin

Austin, TX 78712

PHONE:  (512) 471-3241