BASHDK

BashDK is an in-memory driver patcher that is designed to patch

DKDRIVER in memory so that DKDRIVER will properly control jukebox

disks as normal VMS disks. The patches are minimal and do not

interfere with normal operation on normal VMS disks on SCSI

busses.

The BASHDK program is VMS version specific, so that it works

correctly only where DKDRIVER does not change. It does some testing

to ensure it has the correct DKDRIVER version and will do nothing

if it finds out it has the wrong version.

Use:

Suppose you have an HP jukebox connected to SCSI unit 0 of

bus A.

1. Boot VMS normally.

2. Run BASHDK from a fully priv'd account (needs and uses $cmkrnl)

3. Issue SYSGEN connect commands like

$ sysgen:==$sysgen

$ sysgen connect dka0:/driver=dkdriver

Now the DKA0: device is connected and can be used as a normal VMS

disk.

Internals:

Several locations in DKdriver are patched.

In routine MODE_SENSE:

The instruction

BITB    #<ERR_RECOV$M_AWRE!-    ; The following four flags must have

ERR_RECOV$M_ARRE!-    ; default values of zero: automatic

ERR_RECOV$M_RC>,-     ; write relocation, automatic read

UCB$B_RECOV_PAR+2(R3)   ; disable correction

is patched so it is a bicb2 followed by changing the branch to always

act as though the bits are zero,

disabling these checks.

;In the early version,also the instruction

;

;        BICL2   #<UCB$M_NOREASSIGN!-    ; Assume REASSIGN_BLOCK supported,

;                  UCB$M_FLOPPY!-        ; Assume not a flexible diskette,

;                  UCB$M_FORMAT>,-       ; Assume FORMAT unsupported

;                UCB$L_DK_FLAGS(R3)

;

;was edited so that it does NOT clear the ucb$m_noreassign flag (which is

;set by the patch to INQUIRE).

This is not done in the production version.

Also in mode_sense:

; If this is a read-only device such as a CDROM, it may not have real tracks,

; sectors, and cylinders. If this is the case, READ_CAPACITY will set up a

; dummy values for the geometry parameters. For now, assume success.

92$:    BBS     #DEV$V_SWL,-            ; If this is a read-only device, it's n$

UCB$L_DEVCHAR(R3),75$   ; required to have non-zero values of

The above instruction is changed to test the DEV$V_IDV bit (input device)

which is basically always set. This allows any device to get its geometry

later on instead of giving up with an error.

Other things called from io_packack: are patched.

Subroutine INQUIRY:

BNEQ    40$                     ; Branch if not, illegal device type

BISL    #UCB$M_CDROM,-          ; Indicate CDROM-like device

UCB$L_DK_FLAGS(R3)      ;

BISL2   #UCB$M_HWL,-            ; This a read-only device so set

UCB$L_DK_FLAGS(R3)      ; the hardware write-lock bit

BBSS    #DEV$V_SWL,-            ; This a read-only device so set

UCB$L_DEVCHAR(R3),10$   ; the software write-lock bit

This has been changed to:

blss	40$		; if type < 5 (5=cdrom), not a disk.

; leaves types 5, 6, and 7 treated as disks

; 5 = cdrom, 7 = jukebox.

bisl #<ucb$m_cdrom>,-

ucb$l_dk_flags(r3)	; Set cdrom type (in case true),

; don't support block reassign and say no host based

; shadow support so no read-long gets tried

bisl	#0,ucb$l_dk_flags(r3)	; disable read-only set

bbss	#dev$v_idv,ucb$l_devchar(r3),10$	; set idv bit which

; basically is always set anyway; no-ops this inst.

Also we change the movb #3,<somewhere-ucb-retry-byte>(r5)

to a CVTFB #63,<somewhere-ucb-retry-byte> to change the retry

count from 3 to 120, and we clear the err_recov$m_rc but from

the error recovery ignore mask so that that bit IS cleared

during error recovery and treated as a don't care bit. This bit

is removed from a location in the mode_sense routine also.

The retry count patch is that we change:

IO_RW_RETRY:

MOVB    #RW_RETRY_CNT,-         ; Initialize read/write retry count

UCB$B_RW_RETRY(R3)      ;

to

IO_RW_RETRY:

CVTFB   #63,-         ; Initialize read/write retry count

UCB$B_RW_RETRY(R3)      ;

The two err_recov$m_rc patches are:

One in MODE_SENSE_CHANGABLE where we add the err_recov$m_rc

bit to the instruction:

BICB    #<ERR_RECOV$M_EEC!-     ; Clear don't cares

ERR_RECOV$M_DCR!-     ;

ERR_RECOV$M_DTE>,R2   ;

and again in mode_select:

we remove the bit from the instruction:

BICB3   #^C<ERR_RECOV$M_TB!-    ; Get the error recovery flags of

ERR_RECOV$M_DTE!-   ; interest (the other flags are either

ERR_RECOV$M_PER>,-  ; don't cares or have already been

UCB$B_RECOV_PAR+2(R3),- ; checked for zero values in

R1                      ; MODE_SENSE)

These changes allow the optical disks to be tested for support for

revectoring, but greatly increase the error retry count before

this happens and causes the driver not to treat the err_recov$m_rc

flag, common on opticals, as a fatal error condition. Should the

error result in no data being transferred in the first sector

however, the driver will still report an error. It is for that

reason that the jgdriver intercept function exists, to reissue the

IRP to the DKdriver again when this happens to recover. Once this

happens, opticals will have some errors logged when ECC recovery

happens, but this is essentially always recovered from. The optical

disk signal is noisy, so ECC errors on optical are common, but disabling

reporting of them for optical disk would interfere with reporting

this class of errors for other SCSI disks, where they are serious.

Accordingly, customers need to understand that the error count on

their optical disks will be usually nonzero, but that it is not

meaningful, and their applications will not see these errors. If we

had a hardware board, or a totally new driver, we could avoid

these bogus error reports, but as we're using the DEC driver in order

not to mess up access to other SCSI devices, these error reports

will occur and just need to be ignored.

Note however that a second test in mode set will still set the readonly

bits (dev$m_hwl and dev$m_swl) so long as the SCSI config reading

commands return writelock. My tests show this happening for CDroms.

Even if this isn't done, one can still use mount/nowrite to prevent

attempts at write access to CDs, though it will (obviously) do no harm.

These changes can obviously be made on future DKdriver versions.

While this change means dkdriver could attempt to access type 6 SCSI

devices, in fact this doesn't generally happen because one must

use the sysgen connect DKcnnn:/driver=dkdriver command to connect

disks. So long as one doesn't do something stupid like command to

connect one's scanner as a disk, this latent (mis)feature will

cause no harm.

As things now appear, it isn't clear to me that any further patches are

needed. I could place a bit of code out of line in there, but the

few error paths changed are basically operational errors, causing

no more problems because other errors a bit later will be seen (as

in the case of no geometry...mount will just fail a bit later if

no geometry is finally available, for example).

Glenn C. Everhart

2/15/1994

3/2/1994