BASHDK

Written by Glenn Everhart for Acorn Software, 1994

Released April 1994 by Acorn Software for general use.

BashDK is an in-memory driver patcher that is designed to patch

DKDRIVER in memory so that DKDRIVER will properly control jukebox

disks as normal VMS disks. The patches are minimal and do not

interfere with normal operation on normal VMS disks on SCSI

busses.

The BASHDK program is VMS version specific, so that it works

correctly only where DKDRIVER does not change. It does some testing

to ensure it has the correct DKDRIVER version and will do nothing

if it finds out it has the wrong version. The program currently

has no licensing code. Since it is VMS version specific, it may

be enough to have code that only writes the license owner name

to opa0: when it runs. Alternatively the full license key code that

Branches uses could be incorporated.

The current version of the DK driver patch system is built for VMS 5.5-x

and use should not be attempted on other VMS versions.

Normally one should run the package by using the BASHDK_SETUP.COM

script. This document describes how to use it by hand however.

Note that it is expected you know the SCSI bus and unit your optical

disk or jukebox is connected to, so that if it's on SCSI bus A

the disk name starts with "DKA" and if on bus B, with "DKB", and

the VMS unit number is 100 times the SCSI unit number, so a disk on

SCSI bus B connected to SCSI unit 3 would be called DKB300: as an

example.

Use:

Suppose you have an HP jukebox connected to SCSI unit 0 of

bus A. This would normally show up as DKA0: if it were a regular SCSI disk.

1. Boot VMS normally.

2. Run BASHDK from a fully priv'd account (needs and uses $cmkrnl)

3. Issue SYSGEN connect commands like

$ sysgen:==$sysgen

$ sysgen connect dka0:/driver=dkdriver

Now the DKA0: device is connected and can be used as a normal VMS

disk.

However, optical disks have a fairly low signal level so that it is

normal for the electronics to use error correction codes (ECC) to

recover errors that appear transiently. This is normal on optical and

not significant, but DKdriver is not designed to handle large numbers

of ECC errors.

To avoid disabling ECC handling on normal SCSI disks, these errors are

handled by an intercept system which is turned on for each optical

disk and which retries the ECC errors where needed, leaving error

handling for normal SCSI disks alone.

Do this by loading and turning on JGAn: units for each optical disk

you use.

$ sysgen:==$sysgen

$ sysgen conn JGA0:/noadapter/driver=JGdriver

$ set comm sys$system:jgctl

$ jgctl/verify JGA0: DKA0:

Thus, to use the package:

1. Copy JGCTL.CLD, BASHDK.EXE, and JGDRIVER.EXE into sys$system:

2. On the node where the optical disk is attached perform the following

steps. Assume the optical disk is on SCSI bus B, unit 2

here for the example. If on bus A you'd use DKA, not DKB, and

if it's on SCSI LUN n you use n00 as the unit number,

where here we'll use "200".

Note: Do this from the system account. You can insert these commands into

systartup_v5.com if you like.

$ sysgen:==$sysgen

$ run sys$system:bashdk

$ sysgen connect dkb200:/driver=dkdriver

$ sysgen connect JGA0:/noadapter/driver=jgdriver

$ set command sys$system:jgctl

$ JGCTL/VERIFY JGA0: DKB200:

(connect other JGAn: units and use JGCTL to turn them on in similar

fashion. Thus if you have a second optical disk connected at SCSI bus A

and SCSI LUN 4 you would add after this:

$ sysgen connect dka400:/driver=dkdriver

$ sysgen connect jga1:/noada/driver=jgdriver

$ jgctl/verify JGA1: DKA400:

This will cause the units DKB200: and DKA400: above to behave as normal VMS

disks. You will see errors logged for them, owing to our not having

disabled ECC handling in DKdriver (as your other disks need it), but

the disks will act normally to your applications. They can be used, and

error rates of up to a fraction of a percent of I/O operations on the

optical disks as seen by SHOW ERRORS can be ignored; your applications

will not see them, and the data on the opticals will be recorded correctly.

Note:

The patches are described by documents supplied here. If you use BashDK

it should work on at least some optical disks in VMS 5.5-x. It will need

to be reissued for other versions. This is at present a Vax-only patch

(though some axp support in the pseudo driver exists). Be aware though that

if you use this, you're on your own for support. Nobody is committing

to support for this, though it worked nicely in some tests we ran

on it.